This file is provided "as is" for use by those interested in the extended

Voice Command sets for Zoom's line of voice capable fax modems.  Use of

this command set is NOT recommended for the casual user or others not

experienced with the terminal command mode of your software.  Technical

Support does not provide any information beyond that contained in this file

and if you need further assistance with the command implementation in

software development efforts we recommend that you contact your nearest

Rockwell International Office.

This document is approximately 53 pages long and contains page breaks for

clearer printing.

8.   VOICE COMMANDS

The modem may be configured in response to AT voice commands to

provide enhanced Adaptive Differential Pulse Code Modulation (ADPCM)

coding and decoding for the compression and decompression of digitized

voice.  ADPCM compression supports the efficient storage of voice

messages, while optional coder silence deletion and decoder silence

interpolation significantly increase compression rates.  The ADPCM Voice

Mode supports three submodes once a voice connection is established

(see #CLS command): Online Voice Command Mode, Voice Receive Mode, and

Voice Transmit Mode.

8.1. VOICE SUBMODES

8.1.1.    Online Voice Command Mode

Online Voice Command Mode is the default Voice submode entered when

the #CLS=8 command is issued, and may also be entered from Voice

Receive Mode or Voice Transmit Mode.  Entry into Online Voice Command

Mode is indicated to the DTE via the VCON message, after which AT

commands can be entered without aborting the telephone line connection.

If the modem is the answerer, it enters Online Voice Command Mode

immediately after going off-hook, and can report instances of DTMF

tones and calling tones to the DTE.  If the modem is the originator, it

enters Online Voice Command Mode based on detection of the ringback

cadence going away, upon expiration of the ringback never came timer,

or upon detection of answer tone, and the modem can report DTMF tones,

answer tones, busy tone, and dial tone to the DTE.  (Note that DTMF

tone reporting is supported in this mode if DTMF reporting is enabled

via the #VTD command.) When this mode is entered as a result of going

off-hook with the D or A command, VCON is always sent to the DTE, after

which the modem accepts commands.  If this mode is entered from Voice

Transmit Mode, the DTE has issued the <DLE><ETX>, and the modem res

ponds with VCON.  If this mode is entered from the Voice Receive Mode

because of a key abort, the modem issues the <DLE><ETX> followed by

VCON.  If the #VLS command has switched in a handset or other device in

place of the telephone line, Online Voice Command Mode is immediately

entered, whereas if the telephone line is selected, a physical

connection with another station must occur before entering this mode.

8.1.2.    Voice Receive Mode

Voice Receive Mode is entered when the DTE issues the #VRX command

because it wants to receive voice data.  This typically occurs when

either recording a greeting message, or when recording voice messages

from a remote station.  In Voice Receive Mode, voice samples from the

modem analog-to-digital converter (ADC) are sent to the ADPCM codec for

compression, and can then be read by the host.  AT commands control the

codec bits-per-sample rate and select (optional) silence deletion

including adjustment of the silence detection period.  In this mode,

the modem detects and reports DTMF, dial tone, busy tone cadence, and

inactivity (periods of silence) as enabled by the #VTD and #VSS

commands, respectively.  The modem can exit the Voice Receive Mode only

via a DTE Key Abort, or via Deadman Timer expiration (S30).

8.1.3.    Voice Transmit Mode

Voice Transmit Mode is entered when the DTE issues the #VTX command

because it wants to transmit voice data.  In this mode, the modem

continues to detect and report DTMF and calling tones if enabled by the

#VTD command.  This mode is typically used when playing back greeting

messages or previously received/recorded messages.  In this mode, voice

decompression is provided by the codec, and decompressed data is

reconstituted into analog voice by the DAC at the original voice

compression quantization sample-per-bits rate.  Optional silence

interpolation is enabled if silence deletion was selected for voice

compression.

8.2. VOICE CAPABILITIES

8.2.1.    Call Establishment - Originate

Directed Originate (Dial as a specific modem type) For most call

originations, it is known ahead of time what type of call is being

attempted, and it is acceptable to disconnect if the remote side of the

connection does not cooperate.  In this case, the modem can be

configured ahead of time with the existing +FCLASS (and +FAA=0) or the

#CLS command to be a data, fax, or voice modem.  For Data and Fax

Modes, the modem subsequently either succeeds with the desired type of

connection, or eventually hangs up.  For the Voice Mode, the DTE has

the option of hanging up if there are indications that the remote

station has not answered in voice, thus implementing a directed

originate for voice.  The following are the three connection type

choices:

Voice:    The modem dials and reports call progress to the DTE,

which reduces to reporting "NO DIALTONE," or "BUSY." The modem allows

the DTE to program a time period, which if elapsed after any ringback

is detected, forces the modem to assume the remote has gone off-hook.

A secondary time period (safety valve) can define a maximum elapsed

time after dialing for receiving no ringback before the modem assumes

that the remote has gone off-hook.  This safety valve is devised in

case the remote picks up the telephone before any ringback is

generated, and no other tones are detected.  In this mode, the modem is

attempting to make a voice connection only and therefore, while

waiting for ringback to disappear, it is also feasible to disconnect

upon detection something which is definitely not voice from the remote,

such as any answer tone.  The modem provides detection of "ringback

went away or never came".

Fax:    The modem dials and reports call

progress to the DTE as in all modes.  A fax Class 1 or Class 2

handshake is pursued according to the current configuration.

Data:    The

modem dials and reports call progress to the DTE as in all modes.  A

data handshake is pursued according to the current configuration.

Adaptive Originate (Dial with Voice/Data/Fax Discrimination) The DTE

may wish to originate a call which adapts to the remote answerer.  For

instance, the user may wish to send a voice message if a human picks up

the telephone, but a facsimile if a fax machine answers.  The modem can

facilitate this type of adaptive originate by extending what it does

for the directed originate modes.  After determining that the remote

station has picked up the line, the modem goes back to Online Voice

Command Mode, thus terminating the "connecting state." Once in this

mode, the modem reports what it receives from the answerer via specific

result codes to the DTE.  The DTE can then have the option of pursuing

a data, fax, or voice connection.

8.2.2.    Call Establishment - Answer

Directed Answer (Answer as a specific modem type) If the DTE wants to

be only one kind of answerer (i.e., voice, fax, or data), it can

configure the modem to answer exclusively in the chosen mode.

Voice

The modem is configured to answer in Voice Mode only and assumes the

caller will cooperate.  After going off-hook, the voice VCON is issued,

no answer tone is generated, and the modem is immediately placed in

Online Voice Command Mode.  The DTE typically responds by sending a

greeting message of some type, and DTMF tone recognition/reporting can

be enabled.  Eventually, an incoming voice message can be recorded by

the host.  (Unpredictable results occur if the caller is not prepared

for a voice call.)

Fax    The modem is configured to answer in Class 1 or

Class 2 Fax Mode only, and it assumes the caller is going to cooperate.

This configuration has the effect of disabling Voice Mode, forcing

+FCLASS to either 1 or 2, and forcing both +FAA and +FAE to 0.

Data

The modem is configured to answer in Data Mode only and assumes the

caller is going to cooperate.  This configuration has the effect of

disabling Voice Mode, forcing +FCLASS = 0, and forcing both +FAA and

+FAE to 0.

Adaptive Answer (Answer with Voice/Data/Fax Discrimination)

In normal operation, it is desirable for a modem supporting fax and

voice to provide the ability to discriminate between the two when

answering unsolicited or unattended calls.  (It is most often the case

that a fax is received or a voice message recorded when nobody is

present.)

Data/Fax Discrimination    If the DTE wishes to allow for a data

or fax call, the +FCLASS and +FAA or +FAE commands can be configured

for adaptive answer between data and Class 1 or Class 2 fax.

Voice/Fax Discrimination

This is the most important discrimination capability

needed from the user's standpoint.  The modem must be configured for

Voice (#CLS=8), causing the modem to enter Online Voice Command Mode

immediately upon going off-hook.  In Voice Mode, the DTE automatically

receives indications of DTMF tones and Calling Tones.  The DTE can now

switch to Voice Transmit Mode in order to play a greeting message,

perhaps one which instructs the caller how to enter specific DTMF

sequences to switch modes.  The DTE can then react to the response, or

the lack thereof, to such a message.  The modem supports switching to a

Class 1 or Class 2 answer mode by virtue of the #CLS=1 or 2 commands,

and if such a switch is made and fails, the modem reports the failure

but does not hang up, allowing the DTE further experimentation time.

If the user wishes to switch to Class 1 or 2, but also wants the DTE to

indeed hang up the line if the fax fails, the +FCLASS command should be

used instead of the #CLS command.  The only difference between these

commands is that issuing +FCLASS cancels the modem's memory of voice,

whereas #CLS causes the modem to remain off-hook, even if a fax or data

handshake fails, until it receives an H command.

Voice/Data/Fax Discrimination

The DTE can try data modem operation after an answer by

changing the #CLS setting to 0.  A data handshake attempt can be added

based upon DTMF responses or lack thereof.

8.2.3.    Voice Data Transfer

A significant area of concern when handling the transfer of voice

data is the data transfer rate on the modem/DTE interface.  Data

transfer rates can be expressed as the number of interrupts which must

be serviced per time period to keep up.  This is a function of the

sampling rate and compression method (if any) used by the modem, and

the DTE interface speed required to handle the data flow on the

telephone line side.  Table 8-1 demonstrates the relationship between

the sampling rate, interrupt rate, and DTE speed necessary for the

modem to support various compression ratios.  The 16C550 UART is

strongly recommended to support Voice Mode operation, and is essential

for M S Windows environments.

Table 8-1.  7.2 KHZ Sample Rate DTE Speeds

Modem Samples per 8-bit Byte

Required Asynchronous  DTE Speed (8N1 Format)

Implied DTE Speed Supported

2 (4-bits)

36,000 bps

38.4 kbps

2.66 (3-bits)

27,000 bps

38.4 kbps

4 (2-bits)

18,000 bps

19.2 kbps

Note: The current exclusive sampling rate is 7.2 kHz.

8.2.4.    Tone and Status Monitoring_ Shielded <DLE> Statuses

The modem can detect specific tones and other status information, and

report these to the DTE while in any of the three voice submodes.  The

modem simultaneously looks for 1300 and 1100 Hz calling tones when

answering, and for CCITT and Bell answer tones when originating.  The

modem can also detect dial or busy tones in any of the three voice

submodes.  All detected tones, as well as certain other statuses

addressed in Table 8-2 such as silence and "telset off-hook" (i.e.,

handset off-hook) are reported as shielded codes.  When in Online

Voice Command Mode or Voice Transmit Mode, the codes in Table 8-2 are

sent to the DTE immediately upon verification by the modem of the

associated tone, status, or cadence.  In this mode, the 2-character

code is not buffered, nor does the DTE have the ability to stop the

code with flow control.  If the DTE has started (but not completed)

sending any AT command, the Tone Monitoring function is disabled until

the command has been received and processed.  The modem can

discriminate between single and multiple DTMF tones received.  If

calling tone, dial tone, busy tone, or answer tone is detected, this

detection is reported repeatedly (at reasonable intervals) if the DTE

takes no action, and the tone continues to be detected.

Table 8-2. Shielded Codes Sent to the DTE

Code Sent to DTE

Meaning

<DLE>0 - <DLE>9, <DLE>*, <DLE>#, <DLE>A -<DLE>D

DTMF.  Digits 0 through 9, *, #, or A through D detected by the

modem, i.e., user has pressed a key on a local or remote telephone.

The modem sends only one <DLE> code per DTMF button pushed.

<DLE>a

Answer Tone (CCITT).  Send to the DTE when the V.25/T.30 2100 Hz

Answer Tone (Data or Fax) is detected.  If the DTE fails to react to

the code, and the modem continues to detect Answer tone, the code is

repeated as often as once every half second.

<DLE>b

Busy.  Sent in Voice Receive Mode when the busy cadence is detected,

after any remaining data in the voice receive buffer.  The modem sends

the busy <DLE>b code every 4 seconds if busy continues to be detected

and the DTE does not react.  This allows the DTE the flexibility of

ignoring what could be a false busy detection.

<DLE>c

Calling Tone.  Sent when the T.30 1100 Hz Calling Tone (Fax Modem) is

detected in any of the voice operating modes.  The modem assumes that

the calling tone is valid and sends this code only after 4 seconds of

proper cadence has been detected.  If the DTE does not react to the

<DLE>c, and calling tone continues, the code is sent again as often as

once every 4 seconds.

<DLE>d

Dialtone.  Sent in Voice Receive Mode when dial tone is detected

after any remaining data in the voice receive buffer.  The modem sends

the <DLE>j code every 3 seconds if dial tone continues to be detected

and the DTE does not react.  This allows the DTE the flexibility of

ignoring what could be a false dial tone detection.

<DLE>e

European Data Modem Calling Tone.  Sent when the V.25 1300 Hz Calling

Tone (Data Modem) is detected in a voice submode.  The modem assumes

that the calling tone is valid, and sends this code only after 4

seconds of proper cadence has been detected.  If the DTE does not

react to the <DLE>e and calling tone continues, the code is sent again

as often as once every 4 seconds.

<DLE>f

Bell Answer Tone.  Sent when Bell 2225 Hz Answer Tone (Data) is

detected.  If the DTE fails to react to the code and the modem

continues to detect Answer tone, the code is repeated as often as every

1/2 second.

<DLE>h

Hung Up Handset.  Sent immediately when the modem detects that the

local handset has hung-up (i.e., has transitioned from off-hook to

on-hook) while in any of the three voice submodes.

<DLE>o

Overrun.  Sent in Voice Receive Mode if the voice receive data buffer

overflows, in which case, the latest data is lost in favor of retaining

the oldest data in the buffer.  (The DTE may not have been reading data

from the modem lately.) The modem inserts t he <DLE>o at the end of the

buffer, thus marking the location where data was lost by the DTE.  The

modem does not append<E> <D><DLE>o codes to the buffer contiguously.

If more than one data byte is lost before the DTE begins reading again,

there is only on <DLE>o code in the data stream.  However, if the DTE

resumes reading, and then once again allows the buffer to overflow, a

new <DLE>o is appended to the data by the modem.  Thus, the<E>

<D><DLE>o does not indicate how many bytes of contiguous data has been

lost, but may report multiple gaps of lost data in the data stream.

<DLE>q

Quiet.  Sent in Voice Receive Mode following any remaining data in

the receive voice buffer when the silence detection timer (#VSP)

expires and there has been voice data passed to the DTE.  Note that

with #VSS=0, the Quiet message never occurs.  But with silence

deletion turned on, #VSS may not in itself be sufficient since "cracks

and pops" for a short duration directly after a connection should not

be considered voice.  In general, <DLE>q is sent if first there was

sound, and now there is no sound, and means that somebody has stopped

talking, and according to the criteria selected in the #VSP and #VSS

settings, is probably not going to start again.  If the DTE ignores the

<DLE>q code, the modem automatically resets its timer and sends the

code again if the # VSP time period again expires.  This allows the DTE

time to react to what may be a false silence detection.

<DLE>s

Silence.  Sent in Voice Receive Mode after the silence detection

timer (#VSP) expires and if valid voice has not been detected (#VSS).

In general, <DLE>s is sent if no sound was detected at all.  This

differs slightly from Quiet (<DLE>q) in that it can be used by the DTE

in its discrimination algorithms as a means of determining if anything

at all is coming from the remote station.  If the DTE ignores the

<DLE>s code, the modem automatically resets its timer and sends the

code again if the #VSP time period again expires.  This allows the DTE

time to react to what may be a false detection of silence.

<DLE>t

Handset Off-Hook.  Sent one time when the local handset transition

from on-hook to off-hook is detected in any of the three voice

submodes.

<DLE>u

Underrun.  Sent in Voice Transmit Mode when the voice transmit buffer

becomes empty without receiving a <DLE><ETX> or <DLE><CAN> command

first.  The modem remains in transmit mode, but issues the <DLE>u to

indicate that silence is being generated.  When the DTE resumes sending

bytes, the modem allows a small number of bytes to build up in the

buffer before re-enabling the transmitter to avoid repeated underruns.

However, with the underrun condition, the modem does not wait for the

XOFF threshold (or timer) to be met before resuming, as it does on an

initial transmit or resume operation because the remote listener may be

unaware of the underrun, and keeping the silence short may serve to

mask the underrun altogether.

<DLE><ETX>

End of Stream.  <DLE><ETX> is sent to denote the end of a voice data

stream, similar to fax Class 1 and 2.  This occurs in Voice Receive

Mode only after the DTE issues the key abort.

8.2.5.    Shielded <DLE> Commands from the DTE

Most commands issued for voice are implemented as extensions to the

AT command set, and are accepted only if the modem is in a valid

command mode, such as Online Voice Command Mode.  However, there are

some operations which are available to the DTE without necessitating a

switch to Online Voice Command Mode.  The most obvious of these is the

<DLE><ETX> which the DTE issues during Voice Transmit Mode to terminate

an output message.  Table 8-3 lists shielded <DLE> commands recognized

by the modem.

8.2.6.    Voice Record

To effect recording of a message received via a handset or

microphone, the DTE must configure the modem for Voice Mode (#CLS=8),

and select the proper relay setup (#VLS) to instruct the modem whether

to use the auxiliary device.  The modem responds to the #VLS command by

issuing a relay activate command to select the input device.  When a

device other than the telephone line is selected, the modem immediately

enters Online Voice Command Mode (indicated by VCON).  DTMF detection

is therefore enabled as soon as the DTE selects the device, such as a

handset, although the user still needs to physically pick up the

telephone and "press buttons." Even if the DTE has not entered Voice

Receive or Transmit Modes (#VTX or #VRX), these DTMF tones are

delivered via shielded codes identically to when a physical telephone

connection exists but the DTE has not yet commanded receive or

transmit.  When the DTE decides to record the message, it issues the

#VRX command and upon command receipt, the modem immediately switches

to Voice Receive Mode.  Since the microphone or handset is switched in,

the modem immediately issues the CONNECT message indicating that the

modem has switched to Voice Receive Mode and is beginning the transfer

of ADPCM voice data as well as shielded DTMF tones, and other tones and

statuses.  In this recording state, the modem (if so configured)

monitors for line silence.  If the #VSP period expires, the modem sends

the <DLE>q or <DLE>s code to the DTE.

Recording can be canceled only via two events:

1.   The DTE sends any character to the modem. This is the key abort,

and is the normal method of terminating the receive mode.

2.   The S30 deadman timer expires, which is generally an undesired

event.

Table 8-3.  Shielded DTE Codes

Code Sent to Modem

Meaning

<DLE><ETX>

Terminate.  Sent during Voice Transmit Mode to indicate that the DTE

has finished transmitting a voice message.  The modem completes

transmission of any remaining data in the voice transmit buffer before

responding with the VCON message and entering Online Voice Command

Mode.  (This command is ignored by the modem when not in Voice Transmit

Mode.)

<DLE><CAN>

Cancel.  Sent during Voice Transmit Mode to indicate that the DTE has

finished transmitting a voice message and wants the modem to discard

any remaining data in the voice transmit buffer.  The modem immediately

purges its buffer, and then responds with the VCON message entering

Online Voice Command Mode.  (This command is ignored by the modem when

not in Voice Transmit Mode.)

<DLE>p

Pause.  Sent during Voice Transmit Mode to force the modem to suspend

sending voice data to the selected output device(s).  Any data

currently in the voice transmit buffer is saved until either a resume

(<DLE>r), or cancel <DLE><CAN>, is received, in which case the data is

lost.  If a <DLE><ETX> is received during the paused state, the modem

processes it normally, and also automatically resumes transmission of

the data left in the buffer (appended with <DLE><ETX>).  Any other data

received from the DTE while in this paused state is placed in the

transmit buffer according to available space, with flow control active.

(This command is ignored in other voice submodes.)

<DLE>r

Resume.  Sent during Voice Transmit Mode to force the modem to resume

sending voice data to the selected output device(s).  Any data

currently in the voice transmit buffer is now played.  (This command is

ignored in other voice modes.) The pause and resume commands are

different than a simple suspension of data; data in the buffer is

"frozen" and not played when the DTE pauses.

8.2.7.    Voice Playback

To effect playback of a message recorded via a handset or microphone,

or of a message recorded during a voice call, the DTE must configure

the modem for Voice Mode (#CLS=8) and select the proper relay setup

(#VLS) to instruct the modem whether to use the handset or speaker.

The modem responds to the #VLS command by issuing a relay activate

command to select the input device.  The hardware must provide a means

of selecting a handset and/or microphone instead of the telephone line,

as this input device.  When a device other that the telephone line is

selected, the modem immediately enters Online Voice Command Mode

(indicated by VCON).  DTMF detection is thus enabled as soon as the DTE

selects the device, such as a handset, although the user still needs to

physically pick up the telephone before he can issue DTMF tones.  Once

selected, however, the user can indeed pick up the telephone and "press

buttons." Even if the DTE has not entered Voice Receive or Transmit

Modes (#VTX or #VRX), these DTMF tones are delivered via shielded

codes, identically to when a physical telephone connection exists but

the DTE has not yet commanded receive nor transmit.  When the DTE

decides to play the message, it issues the #VTX command, and the modem

immediately switches to Voice Transmit Mode.  Since the speaker or

handset is already switched in, the modem immediately issues the

CONNECT message indicating that the modem is in Voice Transmit Mode

and is expecting voice data from the DTE.  A subsequent <DLE><ETX> has

to be issued to switch back to Online Voice Command Mode.

8.2.8.    Voice Call Termination

Local Disconnect

The DTE can disconnect from a telephone call by commanding a mode

change to Online Voice Command Mode (if not already in it), and by

issuing the H command.

Remote Disconnect Detection

When in Voice

Receive Mode, the modem sends the proper shielded <DLE> code when loop

break, dial tone, or busy tone is detected.  The modem stays in Voice

Receive Mode, however, until the DTE issues a key abort to force Online

Voice Command Mode.  The DTE must issue the H command if it wishes to

hang up.

8.2.9.    Mode Switching

Voice to Fax

If the modem is in Online Voice Command Mode (i.e., it has gone

off-hook with #CLS=8 in effect), the DTE can attempt a fax handshake by

setting #CLS=1 or #CLS=2, followed by the A or D command corresponding

to fax receive or send.  This has the effect of beginning a fax Class

1 or Class 2 handshake (see #CLS command).  Unsuccessful Fax Connection

Attempt to Voice A fax handshake which does not succeed, attempted as

the result of the DTE modifying the #CLS setting from voice (8) to fax

(1 or 2), does not result in the modem hanging up, allowing the DTE the

flexibility of commanding a switch back to Voice Mode with #CLS=8.

Voice to Data If the modem is in the Online Voice Command Mode, the DTE

can attempt a data handshake by setting #CLS=0 followed by the A or D

command.  This has the effect of beginning a Data Mode handshake

according to the current Data Mode S-register and command settings.

Unsuccessful Data Connection Attempt to Voice A data handshake which

does not succeed, attempted as the result of the DTE modifying the #CLS

setting from voice (8) to data (0), does not result in the modem

hanging up, allowing the DTE the flexibility of commanding a switch

back to Voice Mode with #CL S=8.

8.2.10.   Caller ID

The modem supports Caller ID by passing the information received in

Bell 202 FSK format to the DTE after the first RING detect.  The modem

supports both formatted and unformatted reporting of Caller ID

information received in ICLID (Incoming Call Line ID) format as

supported in certain areas of the U.S.  and Canada.  The DTE enables

this feature via the #CID command.

8.3. AT VOICE COMMAND SUMMARY

Table 8-4 provides a complete summary of the AT voice commands

described in detail in Sections 8.3.1 and 8.3.2.

8.3.1.    Global AT Command Set Extensions

The AT commands in the following section are global, meaning that

they can be issued in any appropriate mode (i.e., any #CLS setting).

For consistency, the command set is divided into action commands and

parameters (non-action commands).  Those commands which are action

commands (i.e., those which cause some change in the current operating

behavior of the modem) are identified as such, and the remaining

commands are parameters.

ATA - Answering in Voice

The answer action command works analogously to the way it works in

Data and Fax Modes except for the following:

1.   When configured for Voice Mode (#CLS=8), the modem enters Online

Voice Command Mode immediately after going off-hook.  When the

#CLS=8  command is issued, the modem can be programmed to look

for 1100 and 1300 Hz calling tones (see #VTD), thus eliminatng the

need to do so as part of A command processing.  After the VCON

message is issued, the modem re-enters Online Voice Command Mode

while sending any incoming DTMF or Calling Tone indications to the

DTE as described in Section 8.2.5.

2.  After answering in Voice Mode (#CLS=8), the DTE, as part of its

call discrimination processing, can decide to change the #CLS

setting to attempt receiving a fax in Class 1, or to make a data

connection. In such a case, the DTE commands the modem to proceed

with the data or fax handshake via the A command even though the

modem is already off-hook.

Result Code:

VCON Issued in Voice Mode (#CLS=8) immediately after going off-hook.

Table 8-4.  AT Voice Commands

Command

Function

A

Answering in Voice Mode.

D

Dial command in Voice Mode.

H

Hang up in Voice Mode.

Z

Reset from Voice Mode.

#BDR

Select baud rate (turn off autobaud).

#CID

Enable Caller ID detection and select reporting format.

#CLS

Select data, fax, or voice.

#MDL?

Identify model.

#MFR?

Identify manufacturer.

#REV?

Identify revision level.

#VBQ?

Query buffer size.

#VBS

Bits per sample (ADPCM).

#VBT

Beep tone timer.

#VCI?

Identify compression method (ADPCM).

#VLS

Voice line select (ADPCM).

#VRA

Ringback goes away timer (originate).

#VRN

Ringback never came timer (originate).

#VRX

Voice Receive Mode (ADPCM)

#VSD

Silence deletion tuner (voice receive, ADPCM).

#VSK

Buffer skid setting.

#VSP

Silence detection period (voice receive, ADPCM).

#VSR

Sampling rate selection (ADPCM).

#VTD

DTMF/tone reporting capability.

#VTX

Voice Transmit Mode (ADPCM).

ATD - Dial Command in Voice

The dial action command works analogously to the way it works in Data

or Fax modes.  When in Voice Mode (#CLS=8):

1.  The modem attempts to determine when the remote has picked up the

telephone line and once this determination has been made, the VCON

message is sent to the DTE.  This determination is initially made based

upon ringback detection and disappearance.  (Se e #VRA and #VRN

commands.)

2.  Once connected in Voice Mode, the modem immediately enters the

command state and switches to Online Voice Command Mode which enables

unsolicited reporting of DTMF and answer tones to the DTE.

Parameters:    Same as Data and Fax modes.

Result Codes:

VCON Issued in Voice Mode (#CLS=8) when the modem determines that the

remote modem or handset has gone off-hook, or when returning to the

Online Voice Command Mode.  (See #VRA and #VRN.)

NO ANSWER Issued in Voice Mode (#CLS=8) when the modem determines

that the remote has not picked up the line before the S7 timer expires.

ATH - Hang Up in Voice

This command works the same as in Data and Fax modes by hanging up

(disconnecting) the telephone line.  There are, however, some specific

considerations when in Voice Mode:

1.  The H command forces #CLS=0, but does not destroy any of the

voice parameter settings such as #VBS, #VSP, etc.  Therefore, if the

DTE wishes to issue an H command and then pursue another voice call, it

must issue a subsequent #CLS=8 command, but it needn't reestablish the

voice parameter settings again unless a change in the settings is

desired.

2.  The #BDR setting is forced back to 0, re-enabling autobaud.

3.  If the #VLS setting is set to select a device which is not, or

does not include the telephone line (such as a local handset or

microphone), the H command deselects this device and reselects the

normal default setting (#VLS =0).  Normally, the DTE shou ld not issue

the H command while connected to a local device such as a handset,

because merely selecting this device results in VCON.  The normal

sequence of terminating a session with such a device is to use the #VLS

command to select the telephone line, which by definition makes sure it

is on-hook.

Parameters:    Same as Data and Fax modes.

Result Codes:  Same as Data and Fax modes.

ATZ - Reset from Voice Mode

This command works the same as in Data and Fax modes.  In addition,

the Z command resets all voice related parameters to default states,

forces the #BDR=0 condition (autobaud enabled), and forces the

telephone line to be selected with the handset on-hook.  No voice

parameters are stored in NVRAM so the profile loaded does not affect

the voice aspects of this command.

Parameters:    Same as Data and Fax modes.

Result Codes:  Same as Data and Fax modes.

#BDR - Select Baud Rate (Turn off Autobaud)

This command forces the modem to select a specific DTE/modem baud

rate without further speed sensing on the interface.  When a valid

#BDR=n command is entered, the OK result code is sent at the current

assumed speed.  After the OK has been sent, the modem switches to the

speed indicated by the #BDR=n command it has just received.  When in

Online Voice Command Mode and the #BDR setting is nonzero (no autobaud

selected), the modem supports a full duplex DTE interface.  This means

that the DTE can enter commands at any time, even if the modem is in

the process of sending a shielded code indicating DTMF detection to

the DTE.  When in Online Voice Command Mode and the #BDR setting is

zero (autobaud selected), shielded code reporting to the DTE is

disabled.  [Note that when #BDR has been set nonzero, the modem employs

the S30 Deadman Timer, and this timer starts at the point where #BDR is

set nonzero.  If this period expires (nominally 60 seconds) with no

activity on the DTE interface, the modem reverts to #BDR=0 and #CLS=0.]

Parameters:    n = 0 - 48 (New baud rate is n*2400 bps)

Default:  0

Result Codes:

OK        If n = 0 - 48.

ERROR     Otherwise.

Command options:

#BDR?  Returns the current setting of the #BDR command as an ASCII decimal

value in result code format.

#BDR=?    Returns a message indicating the speeds that are supported.

#BDR=0    Enables autobaud detection on the DTE interface.

#BDR=n    Where 1<n<48.  Sends OK message at current speed, then

switches to the new speed defined by n*2400 bps unless and until

another #BDR=n command is received.  Autobaud is disabled, and

the character format is maintained at the format most recently

detected.

#CID - Enable Caller ID Detection and Select Reporting Format

This command selects or disables Caller ID recognition and reporting

in any mode.

Parameters:    n = 0, 1, or 2

Default:  0

Result Codes:

OK   n = 0, 1, or 2.

ERROR     Otherwise.

Command options:

#CID?  Returns the current setting (0,1 or 2) of the #CID command as

an ASCII decimal value in result code format.

#CID=? Returns the message, "0,1,2".

#CID=0 Disables Caller ID.

#CID=1 Enables formatted Caller ID reporting of ICLID SDM (Single Data

Message)  and MDM (Multiple Data Message) packets.

#CID=2 Enables unformatted Caller ID reporting of any ICLID packet received

after the first RING cycle

including SDM, MDM, or call waiting packets.

#CLS - Select Data, Fax,

or Voice This command selects Data, Fax, or Voice Mode from operation

in any mode.  Parameters: n = 0, 1, 2, or 8 Default: 0 Result Codes: OK

If n = 0, 1, 2, or 8.  ERROR Otherwise.  Command options:

#CLS? - Returns the current setting (0,1,2, or 8) of the #CLS command as an

ASCII decimal value in result code format.  #CLS=?  Returns the

message, "0,1,2,8".

#CLS=0 Data. This is similar to

setting +FCLASS=0, and instructs the modem to act like a data modem on

subsequent answer or originate operations.  When a disconnect or

inactivity time out in the non-autobaud mode is detected, the modem

automatically sets the #CLS setting to 0 and hangs up.  This ensures

that the modem is always in a known state despite disorderly DTE

behavior.

#CLS=1 Class 1 fax. This is similar to setting +FCLASS=1,

and instructs the modem to be a Class 1 fax modem.  Once this is set,

either the +FAA or +FAE command can be used to force subsequent answers

to be Class 1 adaptive answers.

#CLS=2 Class 2 fax. This is similar to setting +FCLASS=2, and instructs the

modem to be a Class 2 fax modem.  Once this is set, the +FAA command

can be used to force subsequent answers to be Class 2 adaptive answers.

#CLS=8 Voice Mode. This is the main setting the DTE uses to effect

directed or adaptive answer or originate sequences involving voice

modes.  All telephone calls initialized by #CLS=8 result (after answer

or successful call progress) in the modem in Online Voice Command Mode.

#MDL? - Identify Model. This command requests the model number or name of

the modem.  (The response returned may be altered by the OEM via the

ConfigurACE utility program.)

Command option:

#MDL? -  RC96V24AC

#MFR? - Identify Manufacturer This command requests the modem manufacturer.

(The response returned may be altered by the OEM via the ConfigurACE

utility program.)

Command option:

#MFR? -  ROCKWELL

#REV?  - Identify Revision Level This command requests the revision level

of the modem.  (The response returned may be altered by the OEM via

the ConfigurACE utility program.)

Command option:

#REV?  - XXXX-XXXX-XXX YYY (Those places occupied by "X"'s represent the

part number, while those places occupied by `Y"'s represent the revision

number.  The values are determined at time of production.)

8.3.2. AT#V Commands Enabled Only in Voice Mode (#CLS=8)

The commands described in the following subsection are extensions to

the command set which the modem recognizes only when configured for

Voice Mode with the #CLS=8 command.

#VBQ? - Query Buffer Size

This query-only command returns the size of the modem's voice

transmit and voice receive buffers in ASCII decimal, followed by the OK

result code.

Parameters:    None

Command option:

#VBQ?  Returns the size of the modem voice transmit and voice receive buffers.

#VBS - Bits Per Sample (Compression Factor)

This command selects the degree of ADPCM voice compression to be

used.

Parameters:    n = 2, 3, or 4  (Voice compression bits per sample)

Default:  4

Result Codes:

OK   If n = 2, 3, or 4.

ERROR     Otherwise.

Command options:

#VBS?  Returns the current setting of the #VBS command as an ASCII decimal

value in result code format.

#VBS=?  Returns "2,3,4", which are the ADPCM compression bits/sample rates

available.  These bits/sample rates are correlated with the #VCI?

query command response which provides the single compression method

available.

#VBS=2    Selects 2 bits per sample.

#VBS=3    Selects 3 bits per sample.

#VBS=4    Selects 4 bits per sample.

#VBT - Beep Tone Timer

This command defines the time period, in 10ths of a second up to 4

seconds (40) which is used by the modem as the DTMF or fixed tone

duration for generating tones via the D command while in Online Voice

Command Mode.

Parameters:    n = 0 - 40 (0 - 4 seconds)

Default:  10

Result Codes:

OK   If n = 0 - 40.

ERROR     Otherwise.

Command options:

#VBT?     Returns the current setting of the #VBT command as an ASCII

decimal value in result code format.

#VBT=?    Returns the message, "0-40".

#VBT=0    Disables the tone generation capability.

#VBT=1 to 40   Sets tone duration time.

#VCI? - Identify Compression Method

This command queries the modem as to its compression method and raw

bits-per-sample capability.  <OP0C0>;<D> pairs, separated by <LF0>s.

Within each line, the <O> is left justified, and is separated from the

<D> by the semicolon character and separated fro m the bits-per-sample

string by another semicolon.<F255P255C255>

Parameters:    None

Command option:

#VCI?     Returns the message, "ROCKWELL;ADPCM;8".

#VLS - Voice Line Select

This action command can be used to select which devices are routed

through the modem.

Parameters: n = 0, 1, 2, 3, or 4 [This is the device position number

(not device number) as reflected in response to #VLS=?]

Default:  0

Result Codes:

OK   If n = 0, 1, 2, 3, or 4.

VCON If device selected does not connect to the telephone line.  (A speaker

is such a device, but a telephone line with speaker ON is not such a

device, and generates OK.) ERROR If n does not equal 0, 1, 2, 3, or 4,

or if already off-hook.

Command options:

#VLS?  This query returns the current setting of the #VLS command as an

ASCII decimal value in result code format.

#VLS=?  This query requests a report of the device types available

for selection.  The response is a series of numbers separated by

commas, and each number indicates a device position number.  Currently,

device position numbers do happen to correspond to device type numbers

as shown in Table 8-5, but this correlation is not required (i.e., 0's

could be placed in the first few device position number locations to

correspond to multiple telephone line connections).

#VLS=0 The default option on the modem.  This command instructs the

modem that when entering any of the three voice operating submodes

(Online Command, Transmit, or Receive), that the telephone line

interface should be routed through the modem.  The OK response is sent

to the DTE, and any previous connection is lost (i.e., the modem ends

up on-hook as a result of issuing this command to connect to the

telephone line).

#VLS=1 This command instructs the modem to route only the handset through

the modem.  This setting can be chosen before recording a greeting message.

#VLS=2 This command instructs the modem to route only the speaker

through the modem.  This setting can be chosen before playing back any

message.  The modem immediately switches to Online Voice Command Mode ,

and the VCON response is generated for completeness.  However, since

this is an output only device, nothing of consequence can happen until

the DTE sends the #VTX command.

#VLS=3 This command instructs the modem that only the auxiliary input

device (microphone) should be routed through the modem.  This setting

can be chosen before recording a greeting message.

#VLS=4 This command is the same as #VLS =0, except that the modem  enables

the internal speaker as well as the telephone line/handset circuit.

Table 8-5.  Device Types Supported by #VLS

ASCII Digit

Device Type and Considerations

0

Telephone Line with Telephone handset.  This is the default device

selected.  In this configuration, the user can pick up a handset which

is connected to the same telephone line as the modem, and * record both

sides of a conversation with a remote station.  The modem currently

supports one telephone line/handset, which is in the first position of

the #VLS=?  response.  (Note that the modem can interface to multiple

telephone lines by having "0"'s in multiple positions in the #VLS ?

response.) If a telephone line is selected, the modem must be on-hook

or it hangs up.  The OK message is generated.

1

Transmit/Receive Device (other than telephone line).  This is a

handset, headset, or speaker-phone powered directly by the modem.  When

such a device is selected, the modem immediately enters Online Voice

Command Mode, DTMF monitoring is enabled if applicable, and the VCON

response is sent.  The modem supports one such device as the second

device listed in the #VLS=?  response.

2

Transmit Only Device.  Normally, this is the onboard speaker.  When

this device is selected, the modem immediately enters Online Voice

Command Mode, and the VCON response is sent.  The modem supports

selection of the internal speaker as the third device listed in

#VLS= ? response.

3

Receive Only Device.  Normally, this is a microphone.  When such a

device is selected, the modem immediately enters Online Voice Command

Mode, DTMF monitoring is enabled if applicable, and the VCON response

is sent.  The modem supports one microphone as the fourth element

returned in the #VLS =?  response.

4

Telephone line with Speaker ON and handset.  This device type can be

used to allow the DTE to select the telephone line/handset (if picked

up) with the modem speaker also turned ON.  This can be used by the DTE

to allow the user to monitor an incoming message as it is recorded.

#VRA - Ringback Goes Away Timer (Originate)

This command can be used, when originating a voice call (#CLS=8) to

set the "Ringback Goes Away" timer value, i.e., an amount of time (in

units of 100 ms) measured from when the ringback cadence stops once

detected.  If ringback is not detected within this period, the modem

assumes that the remote has picked up the line and switches to Online

Voice Command Mode.  Every time a ringback cycle is detected, this

timer is reset.

Parameters:    n = 0 - 255  (0 - 25.5 seconds)

Default:  70

Result Code:   OK

Result Code:

OK   If n = 0 - 255.

ERROR     Otherwise.

Command options:

#VRA?  Returns the current setting of the #VRA command as an ASCII decimal

value in result code format.

#VRA=? Returns the message, "0-255".

#VRA=0 Turns off the "ringback goes away timer." After one ringback, the

dialing  modem sends VCON and enters Online Voice Command Mode

immediately.

#VRA=n Where n defines the period without ringback (after at least one

ringback has been detected) in 100 ms units.

#VRN - Ringback Never Came Timer (Originate)

This command can be used when originating a voice call (#CLS=8) to

set the "Ringback Never Came" timer value, i.e., an amount of time (in

units of 100 ms) measured from completion of dialing.  If ringback is

not detected within this period, the modem assumes the remote has

picked up the line and switches to Online Voice Command Mode.

Parameters:    n = 0 - 255  (0 - 2.55 seconds)

Default:  100

Result Code:

OK   If n = 0 - 255.

ERROR     Otherwise.

Command options:

#VRN?  Returns the current setting of the #VRN command as an ASCII decimal

value in result code format. #VRN=?    Returns the message, "0-255".

#VRN=0 Turns off the "ringback never came timer." After dialing, the modem

sends VCON and immediately enters Online Voice Command Mode.

#VRN=n Where n defines the period without ringback after dialing in  100 ms

units.

#VRX - Voice Receive

This action command is only valid if the modem is in the Online Voice

Command Mode (indicated previously with the VCON message), and is the

switch to the Voice Receive Mode.  This command is used when a voice

file is to be received from the line, microphon e, or handset.  The

#VLS command should have been previously issued to select the input

source.

Parameters:    None

Result Codes:

CONNECT   When voice transfer from modem to DTE can begin.

ERROR     If #VLS=0 and not connected to any input device.

#VSD - Enable Silence Deletion (Voice Receive)

This command is used to enable or disable Voice Receive Mode silence

deletion.

Parameters:    n = 0 or1

Default:  0 or 1

Result Code:

OK   If n = 1.

ERROR     Otherwise.

Command options:

#VSD?  Returns the current setting of the #VSD command as an ASCII decimal

value in result code format.

#VSD=?  Returns the message, "1".

#VSD=0    Turns off the silence deletion.

#VSD=1 Turns on the silence deletion if the silence sensitivity setting

#VSS is nonzero.  The aggressiveness of the silence deletion is

controlled by #VSS.

#VSK - Buffer Skid Setting

This command queries and sets the number of bytes of spare space,

after the XOFF threshold is reached, in the modem's buffer during Voice

Transmit Mode.  This equates to the "skid" spare buffer space, or the

amount of data the DTE can continue to send after being told to stop

sending data by the modem, before the modem voice transmit buffer

overflows.

Parameter:     n = 255  (Number of bytes of "skid spare buffer space)

Default:  255

Result Code:

OK   If n = 255.

ERROR     Otherwise.

Command options:

#VSK?  Returns the current setting of the #VSK command as an ASCII decimal

value in result code format.

#VSK=?    Returns the message, "255".

#VSK=n n defines the 255 unused bytes remaining in the modem voice transmit

buffer after an XOFF is sent to the DTE.

#VSP - Silence Detection Period (Voice Receive)

This command sets the Voice Receive Mode silence detection period

(inactivity timer) value.  The parameter, in units of 100 ms, can be

used when receiving voice data.  This is an amount of time, which if

elapsed without receiving any ADPCM data, causes the modem to send the

<DLE>s or <DLE>q codes after insuring that the buffer is empty.  There

are two cases:

1.  With the modem's silence deletion feature enabled, the #VSP

setting is in effect an inactivity timer.  The modem must reset this

timer on every byte of voice data received.

2.  With the modem's silence deletion feature disabled, the modem

determines what constitutes silence.  This involves monitoring and

debouncing the modem value for average energy.  If this debounced value

is less than an arbitrary threshold constituting the modem's

definition of silence for a period greater than that defined by the

#VSP setting, the modem sends the <DLE>q or <DLE>s shielded code to the

DTE.

Parameters:    n = 0 - 255 (0 - 25.5 seconds)

Default:  55

Result Code:

OK   If n = 0 - 255.

ERROR     Otherwise.

Command options:

#VSP?  Returns the current setting of the #VSP command as an ASCII decimal

value in result code format. #VSP=?    Returns the message, "0-255".

#VSP=0 Turns off the silence period detection timer.

#VSP=n Where n defines the period without received voice data in 100 ms units.

#VSR - Sampling Rate Selection

This parameter, along with the bits per sample (#VBS) command,

determines the necessary DTE interface speed to transmit and receive in

the Voice Mode.

Parameter:     n = 7200  (7200 Hz sampling rate)

Default:  7200

Result Codes:

OK   If n = 7200.

ERROR     Otherwise.

Command options:

#VSR?  Returns the current setting of the #VSR command as an ASCII decimal

value in result code format.

#VSR=?    Returns the message, "7200".

#VSR=7200 Default.

#VSS - Silence Detection Tuner (Voice Receive)

This command enables or disables the Voice Receive mode silence

detection, and controls the sensitivity employed by the modem in

compressing periods of silence.

Parameters:    n = 0 - 3

Default:  2

Result Codes:

OK   If n = 0 - 3.

ERROR     Otherwise.

Command options:

#VSS?  Returns the current setting of the #VSS command as an ASCII decimal

value in result code format.

#VSS=? Returns the message, "0-3".

#VSS=0 Disables silence detection by the modem when in Voice Receive Mode.

#VSS=1 Least sensitive setting.  When this command is received by the

modem, the system is configured to a state which is least likely to

detect and compress periods of silence, but still able to do so if the

line is really quiet.

#VSS=2 Midrange setting (default).  When this command is received by

the modem, the system is configured to a state which is likely to be

the best overall compromise on normal telephone lines.

#VSS=3 Most sensitive setting.  When this command is received by the

modem, the system is configured to a state which is most likely to

detect and compress periods of silence.

#VTD - DTMF Tone Reporting

This command queries and controls which types of tones can be

detected and reported to the DTE via shielded codes in Voice Transmit,

Voice Receive, and Online Voice Command Modes.

Parameters: i,j,k (three ASCII hexadecimal bit maps)

Default: 3F,3F,3F

Result Codes:

OK If bits supported with #VTD=?  are selected.

ERROR Otherwise.

Command options:

VTD?  Returns the current setting of the #VTD command as an ASCII decimal

value in result code format.

#VTD=?  Returns the tone reporting capabilities of the modem.

#VTD=i,j,k Where i,j,k corresponds (in ASCII hexadecimal) to the  desired

capabilities, as shown in Table 8-6, for Voice Transmit, Voice

Receive, and Online Voice Command Modes, respectively.

Table 8-6.  #VTD Tone Detection/Reporting Bit Settings

Bit    |     Description

---    |     -----------

0

0 = Disable DTMF tone capability;

1 = Enable DTMF tone capability 1.

1

0 = Disable V.25 1300 Hz Calling tone capability;

1 = Enable V.25 1300 Hz Calling tone capability 1.

2

0 = Disable T.30 1100 Hz Facsimile Calling tone capability;

1 = Enable T.30 1100 Hz Facsimile Calling tone capability 1.

3

0 = Disable V.25/T.30 2100 Hz Answer tone capability;

1 = Enable V.25/T.30 2100 Hz Answer tone capability 1.

4

0 = Disable Bell 2225 Hz Answer tone capability;

1 = Enable Bell 2225 Hz Answer tone capability 1.

5

0 = Disable call progress tone and cadence (e.g., busy and dial tone)

capability;

1 = Enable call progress tone and cadence (e.g., busy and dial tone)

capability 1.

6-7

Reserved.

Notes:

1.  The modem detects this tone/cadence, and reports it via a

shielded code to the DTE.

#VTS - Generate Tone Signals (Online Voice Command)

This action command can be issued to play one of more DTMF or other

tones (such as a "beep") if and only if the modem is in the Online

Voice Command Mode.  The modem parses and plays the tones defined in

the parameter in the order listed, and no key abort is accepted.  The

parameter can have three types of elements separated by commas: Dual or

Single Tones: These are represented by a substring enclosed in square

brackets ("[ ]") within the parameter.  Each such substring consists of

three sub-elements corresponding to 2 frequencies in Hz (0, or

200-3000) and a duration (ASCII decimal in units of 100 ms).

Varying DTMF Digits: This is represented by a substring enclosed in curly

braces ("{ }") within the parameter.  Each such substring consists of

two sub-elements corresponding to a DTMF digits (0-9,A-D,*,#), and

alternate durations in units of 100 ms.

Parameters: The tone generation consists of elements in a list with each

element separated by commas.

Result Codes:

OK Command to play tones on currently selected device is accepted.

ERROR Command was not issued during Online Voice Command Mode or string is

grammatically incorrect.

#VTX - Voice Transmit

This action command can be issued if and only if the modem is in the

Online Voice Command Mode (indicated previously with the VCON message),

and is the switch to Voice Transmit Mode.  #VTX is used when a voice

file is to be transmitted to the line, speaker , or handset.  The #VLS

command should have been previously issued to select the output source.

Parameters:    None

Result Codes:

CONNECT   When voice transmission by DTE can begin.

ERROR     If #VLS =0 and output device not connected.

8.4. S-REGISTERS

The following S-register is global, meaning that it can be set in any

appropriate mode (i.e., any #CLS setting).

S30 - Deadman (Inactivity) Timer

Range:    n = 0 - 255

Default:  0 (OFF, which means DTE should usually set it to some value for

Voice)

Command options:

S30=0     Deadman timer off. No matter how long it might continue, the

modem never spontaneously hangs up the telephone line or switches to

autobaud mode as a result of inactivity.

S30=1 to 255   This is the period of time (in seconds), which if expired

causes the modem to hang up the telephone line if it is off-hook and

no data has passed during the period.  The timer is also active

whenever the #BDR setting is non-zero.  In order to avoid a state

where speed sense is disabled (even though the PC can crash, come back

up, and try to issue commands at what should be a supported speed), the

inactivity time-out occurs if there is no data passed on the DTE

interface within the S30 period, even if the modem is on-hook.  DTE

software must not select a nonzero setting for #BDR until it is ready

to establish a telephone call or virtual connection to a speaker or

microphone.  When there is an inactivity time out with #CLS=8, the

modem always forces #CLS=0 and #BDR=0.

8.5. RESULT CODES FOR VOICE OPERATION

VCON

VCON is sent when the modem is configured for voice (#CLS=8), or when

after answering or originating a call, the modem enters the Online

Voice Command Mode for the first time.  Typically, this is immediately

after an off-hook in answer mode, and after ringback ceases in

originate mode.  VCON is also sent when the DTE requests a switch from

Voice Transmit Mode to Online Voice Command Mode by issuing a

<DLE><ETX> to the modem, or when the DTE requests a switch from Voice

Receive Mode to Online Voice Command Mode via the key abort.

CONNECT

CONNECT is sent when switching from the Online Voice Command Mode to

either Voice Receive Mode via the #VRX command, or to Voice Transmit

Mode via the #VTX command.  This message is sent to the DTE to inform

it that it may begin receiving or send ing ADPCM data.

8.6. EXAMPLES OF VOICE OPERATION

See Tables 8-7 through 8-13.

Table 8-7.  Record a Greeting Message

DTE to Modem

Modem to DTE

Description and Interpretation

#CLS=8

Configure the modem for Voice Mode.

OK

The modem confirms command implemented, and is now set up for voice.

#VBS=4

The DTE asks for 4-bit compression.  The DTE needs to work at 38,400

bps, since, after start and stop bits are added, the speed increases to

36,000 bps.

OK

The modem is configured for 4-bit compression.

S30=60

DTE wants to insure that the modem can recover even if DTE dies with

the baud rate set to a speed which cannot be speed sensed.

OK

Modem now has deadman timer set to 1 minute.  If no activity takes

place after a connection, or no DTE activity takes place after #BDR is

changed, the modem reverts to #CLS=0, #BDR=0.

#BDR=16

The DTE knows that compression requires a 38,400 bps speed, so it

forces this as the new speed.  (For this example, lets say that the DTE

was sending all commands, including this one, at 2400 bps.)

OK

This OK message is sent at 2400 bps, but the modem then switches

to 38,400 bps.  All subsequent commands are assumed to be sent at

38,400 bps.  If the DTE does not switch to 38,400 bps after the

OK<CR><LF>, it will not recognize the data stream coming from the

modem.  The S30 timer is now active.

#VSP=20

The DTE changes its own speed to 38,400 bps and sends this command to

set up the silence detection period for 2 seconds.

OK

The modem agrees.  When receiving voice data, 2 seconds of silence

terminates the Receive Mode.  For handset operation, this amounts to a

hang up.

#VSD=1

This enables silence deletion.

OK

The modem confirms command reception.

#VSS =1

The DTE has decided to use level "1" silence detection sensitivity,

meaning it is making it somewhat more difficult to delete and compress

silence for the modem, but expects that when the user has hung up the

handset, the silence is indeed detected.  The DTE has tried to improve

the quality of the message because it knows this is a handset

operation.  Also, since silence deletion has been enabled, the #VSP

setting for silence detection now becomes basically an inactivity

timer.  If silence deletion is turned off here instead of enabled, the

#VSP timer is based upon average energy readings from the modem.

OK

The modem confirms command reception.

#VLS=?

Find out what type of devices are supported by the product.

0,1,2,3,4

OK

The modem tells the DTE that is has a telephone line with handset

(#VLS=0), an input/output device such as a handset or speaker-phone

(#VLS=1), and an output only device such as a speaker.

#VLS=1

Select the handset as the input device to be used by the modem.

VCON

The modem agrees, switches out the telephone line, switches in the

handset, and the modem enters Online Voice Command Mode.  Since this is

a handset, and since the baud rate has been preselected with #BDR, this

also means that DTMF detection and reporting is now enabled.  The user

can pick up the telephone and send tones at this point, which are

delivered as shielded codes to the DTE, although the voice data is not

yet being passed.

#VRX

This tells the modem that the DTE is ready to receive a voice message

at the current compression/sampling settings.

CONNECT

The modem tells the DTE that it can now expect data.  The modem

enters Voice Receive Mode.  DTMF monitoring is still enabled as well as

ADPCM data flow.

<Data>

User says: "Hello, this is me, press * to skip this message.  I'm

happy that you have called, so press 5 to send your fax, or you can

leave your message at the beep." User then hangs up, the message is

buffered to the DTE, and the DTE waits for the message to end.  The DTE

should have some kind of maximum timer running to ensure that the user

won't speak longer than some predetermined limit.  Alternatively, the

DTE can require the user to enter a DTMF tone to terminate transmission

of the message to the DTE.

<DLE>q

The modem has noticed (#VSP time out) that the user has finished.

The modem reports this while staying in Voice Receive Mode.  DTMF

detection and reporting is still active.  The DTE now has the entire

greeting message on disk.

<DLE>*

The user has inadvertently pressed the "*" button, but the DTE is not

interested in this presently.

!

The DTE issues a key abort to switch to Online Voice Command Mode.

<DLE><ETX>

VCON

The modem exits Voice Receive Mode and switches to Online Voice

Command Mode.

#VLS=0

The DTE did not really have to do this, but wants to make sure that

it gets no more DTMF codes from the handset.  Selection of the

telephone line here forces the modem out of Online Voice Command Mode,

which is equivalent to hanging up.

OK

Table 8-8. Playback a Greeting Message

DTE to Modem

Modem to DTE

Description and Interpretation

#CLS=8

Configure the modem for Voice Mode.

OK

The modem confirms command reception, and is now set up for voice.

#VBS=4

The DTE asks for 4-bit compression.  The data from the DTE needs to

travel at 38,400 bps since after start and stop bits are added, the

speed increases to 36,000 bps.

OK

The modem is configured for 4-bit compression.

#BDR=16

The DTE knows that compression requires a 38,400 bps speed, so it

forces this as the new speed.  (For this example, lets say that the DTE

was sending all commands, including this one, at 4800 bps.)

OK

This OK message is sent at 4800 bps, but the modem switches to 38,400

bps.  All subsequent commands are assumed to be sent at 38,400 bps.

The DTE should switch after the <CR><LF> following the OK.  The S30

timer is now active.

#VSD=1

This enables silence insertion.

OK

The modem confirms command reception.(#VSS is unnecessary for

playback.)

#VLS=2

Select the speaker as the output device to be used by the modem.

VCON

The modem confirms command reception, and enters Online Voice Command

Mode.  Since this is an output only device, nothing more can happen

yet.

#VTX

This tells the modem that the DTE is ready to send voice per the

current compression/sampling settings.

CONNECT

The modem tells the DTE that it can now send data.  The modem enters

Voice Transmit Mode and ADPCM data flow begins.

<Data>

The DTE sends data.  The modem waits 1 second before starting to

transmit to the speaker to ensure uninterrupted data flow.  If the

voice transmit buffer runs empty, the transmitter is disabled.

<DLE><ETX>

The DTE has finished its transmission and now instructs the modem to

enter Online Voice Command Mode after emptying the voice transmit

buffer.

VCON

The modem tells the DTE it is done.  If the DTE sends the <DLE><ETX>

before the modem has finished transmitting all the data in its buffer,

the DTE must wait for this response.  (VCON indicates that the DTE can

send more data at any time with #VTX.)

Table 8-9. Answer Call/Play Greeting/Record Message

DTE to Modem

Modem to DTE

Description and Interpretation

#CLS=8

Configure the modem for Voice Mode.  Note that the #BDR setting is

assumed to be zero here, meaning that no time-out occurs forcing

#CLS=0.

OK

The modem agrees, and is now set up for voice.

#VBS=4

The DTE asks for 4-bit compression for greeting message playback.

The DTE needs to work at 38,400 bps since after start and stop bits are

added, the speed increases to 36,000 bps.

OK

The modem is configured for 3-bit compression.

#VSD=1

This enables silence deletion.

OK

The modem confirms command reception.

#VSS =2

Set silence detection sensitivity.

OK

The modem confirms command reception.

#VLS=0

Select the telephone line as the source, ensuring that the modem is

now on-hook.  DTMF detection is now off regardless of the previous

setting of #VLS.

OK

The modem confirms command reception, and it switches in the

telephone line.

RING

Some time later, we receive the first call.  (Note that the modem

also processes Caller ID information at this time.)

#CLS=8

Configure the modem for Voice Mode again.  Although the modem has

already been so configured, it is a good idea to select #CLS=8 directly

following a RING because in normal use, any S30 time-out or hang up

during Voice Mode forces #CLS=0.

OK

The modem agrees, and is now definitely set up for voice.

#BDR=16

The DTE knows that compression requires a 38,400 bps speed, so it

forces this as the new speed.  (For this example, lets say that the DTE

was sending all commands, including this one, at 9600 bps.)

OK

This OK message is sent at 9600 bps, but the modem then switches to

38,400 bps.  All subsequent commands are assumed to be sent at 38,400

bps until either another #BDR command is received or until the S30

timer expires.  The S30 timer is now activated, and the default is 60

seconds.

A

The DTE tells the modem to answer.

VCON

The modem is now in Online Voice Command Mode.  DTMF and calling tone

detection is enabled.

Table 8-9.  Answer Call/Play Greeting/Record Message (Cont'd)

DTE to Modem

Modem to DTE

Description and Interpretation

#VTX

The DTE does not wait for any tones, and tells the modem that it is

ready to send a voice greeting at the current compression and sampling

settings (4-bit/7200).

CONNECT

The modem tells the DTE that it can now send data.  The modem enters

Voice Transmit Mode and DTMF monitoring is still enabled as well as the

ADPCM data flow.

<Data>

The DTE sends greeting message data.  The modem waits up to 1 second

or until XOFF threshold is reached before starting to transmit to

ensure uninterrupted data flow.  If the voice transmit buffer runs

empty, the transmission is disabled.

<DLE><ETX>

The DTE has finished with its transmission, and now instructs the

modem to enter Online Voice Command Mode since this is the telephone

line.

VCON

The modem tells the DTE it is done.  If the DTE sends the <DLE><ETX>

before the modem has finished transmitting all the data in its voice

transmit buffer, the DTE must wait for this response.

#VBT=12

The DTE requests that DTMF digits be sent as "beeps" with the #VTS

command while in Online Voice Command Mode, with each "beep" having a

duration of 1.2 seconds.

OK

The modem confirms command reception

#VTS=#

The DTE sends a "BEEP" to annotate its greeting message.  This beep

is a 1.2 second long DTMF #.  During this period, the no abort is

possible from the DTE, and should not be necessary.

OK

After the modem sends the "BEEP", it sends VCON to inform the DTE

that it is ready to move on.  DTMF pass through is re-enabled.

#VBS=3

The DTE asks for 3-bit compression to recorded message.  The DTE

needs to work at 38,400 bps since after start and stop bits are added

the speed increases to 27,000<N>bps.

OK

The modem is configured for 3-bit compression.

#VRX

The DTE instructs the modem to turn the line around and begin

recording a message from the remote.

CONNECT

The modem sets things up and enables ADPCM data transfer to the DTE

with the current silence deletion setting.

<Data>

ADPCM data is shipped to the DTE with shielded DTMF tones folded in.

The modem is looking for silence, busy, dial tone, and loop break.  The

DTE records everything in its storage media.

<DLE>b

The caller has finished, hung up, and in this case, a busy cadence

was detected before anything else such as expiration of the #VSP

silence detection period.

X

The DTE uses a key abort to respond to a busy detection.

<DLE><ETX>

VCON

The modem is now in Online Voice Command Mode.

H

DTE sends modem to #CLS=0 with #BDR=0.

Table 8-10.  Answer Call/Record Message/Receive Fax

DTE to Modem

Modem to DTE

Description and Interpretation

#CLS=8

Configure the modem for Voice Mode.

OK

The modem agrees, and is now set up for voice.

#VBS=3

The DTE asks for 3-bit compression.  The DTE needs to work at 38,400

bps since after start and stop bits are added, the speed increases to

27,000 bps.

OK

The modem is configured for 3-bit compression.

#VSS=2

Set silence sensitivity.  (We assume #VSD=1.)

OK

The modem confirms command reception.

#VLS=0

This selects the telephone line, and ensures that it is on-hook.

OK

The modem confirms command reception, and switches in the telephone

line.

#CLS=8

This ensure things are set up for voice.

OK

The modem confirms command reception, and is now set up for voice.

RING

Sometime later, the next call is received.  The modem answers.

#BDR=16

The DTE knows that compression requires a 38,400 bps speed, so it

forces this as the new speed.  (For this example, lets say that the DTE

was sending all commands, including this one, at 9600 bps.)

OK

This OK message is sent at 9600 bps, but the modem then switches to

38,400 bps.  All subsequent commands are assumed to be sent at 38,400

bps.  The S30 timer is now active.

A

The DTE answers.

VCON

The modem is now in Online Voice Command Mode.  DTMF and calling tone

detection is enabled.

#VTX

The DTE does not wait for any tones, and tells the modem that it is

ready to send a voice greeting at the current compression and sampling

settings.

CONNECT

The modem tells the DTE that it can now send data.  The modem enters

Voice Transmit Mode and DTMF monitoring is still enabled as well as

ADPCM data flow.

<Data>

The DTE sends greeting message data.  The modem waits 1 second before

starting to transmit to ensure uninterrupted data flow.  If the voice

transmit buffer runs empty, the transmission is disabled.

<DLE><ETX>

The DTE has finished its transmission and now instructs the modem to

enter Online Voice Command Mode.

VCON

The modem tells the DTE it is done.  If the DTE sends the <DLE><ETX>

before the modem has finished transmitting all the data in its voice

transmit buffer, this response is delayed until the buffer is empty.

#VBT=13

The DTE requests that DTMF digits be dialed as "beeps" while in

Online Voice Command Mode, with each "beep" having a duration of 1.3

seconds.

OK

The modem agrees.

#VTS=

[1000,0,20],9,

The DTE sends a "BEEP" to annotate its greeting message.  This beep

is a 2 second long 1000 Hz tone, followed by a 1.3 second long DTMF 9.

During this period, the DTE is locked out.

OK

After the modem sends the "BEEP", it sends VCON to inform the DTE

that it is ready to move on.  DTMF pass through is re-enabled.

Table 8-10.  Answer Call/Record Message/Receive Fax (Cont'd)

DTE to Modem

Modem to DTE

Description and Interpretation

#VRX

The DTE instructs the modem to turn the line around and begin

recording a message from the remote.

CONNECT

The modem sets things up and enables ADPCM data transfer to the DTE

with the current silence deletion setting.

<Data>

ADPCM data is shipped to the DTE with shielded DTMF tones folded in.

The modem is looking for silence, busy, dial tone, and loop break.  The

DTE records everything in its storage media.

<DLE>5

The caller has finished his voice message, but is now insisting that

he be able to send us a facsimile.

X

DTE sends `X' as key abort.  If the user tried to keep talking after

they hit the DTMF 5, anything said was lost.  Whatever he said before

the 5 is on disk.

<DLE><ETX>VCON

The modem obeys key abort.  Receive Mode is cancelled, and the modem

is now in Online Voice Command Mode.  DTMF is still passed through.

#CLS=2

The DTE instructs the modem to pretend no voice activity has taken

place, although the modem does not hang up if the fax fails.

OK

The modem now thinks its a Class 2 Fax modem.  The DTE can now tailor

the +F settings which were not accessible until +FCLASS became 2.

A

Answer tone is generated and prompts the user on the telephone to

press the button on his fax machine to send his fax.  After the

successful fax is received, the modem hangs up.

Table 8-11.  Answer Call, Determine It's a Fax

DTE to Modem

Modem to DTE

Description and Interpretation

RING

Sometime later, the next beta-site calls.

#CLS=8

Configure the modem for Voice Mode.

OK

The modem agrees, and is now set up for voice.

#VBS=3

The DTE asks for 3-bit compression.  The DTE needs to work at 38,400

bps since after start and stop bits are added, the speed increases to

27,000 bps.

OK

The modem is configured for 3-bit compression.

#BDR=16

The DTE knows that compression requires a 38,400 bps speed, so it

forces this as the new speed.

OK

This OK message is sent at 9600 bps, but the modem then switches to

38,400 bps.  All subsequent commands are assumed to be sent at 38,400

bps.  The S30 timer is now active.

#VSS =2

Set sensitivity to normal setting.

OK

The modem confirms command reception.

#VSD=1

This enables silence deletion.

OK

The modem confirms command reception.

A

Finally, the modem answers.

VCON

The modem is now in Online Voice Command Mode.  DTMF and calling tone

detection is enabled.  (Note that alternatively, the DTE can go into

the Transmit or Receive mode and look for either calling tone or

silence as a means of discrimination.) The DTE waits a few seconds to

give a possible fax machine or remote data/fax modem a chance to send

calling tone without being disturbed by a voice greeting.  (In this

example only calling tone is considered, which excludes a person

sending a fax when he hears answer tone.)

<DLE>c

The modem has detected T.30 Calling tone and presents this

information to the DTE.

#CLS=1

The DTE decides to receive the fax, and tells the modem to pretend

that it just went off-hook as a Class 1 Facsimile modem.  The DTE must

be able to handle Class 1 commands from here on.  Note

OK

The modem is now a Class 1 fax modem.

A

The modem pretends that it just got the incoming RING, and modem

handles the fact that it is already off-hook.

Table 8-12.  Adaptive Fax/Data/Voice; Determine Data

DTE to Modem

Modem to DTE

Description and Interpretation

RING

Sometime later, we receive a call.

#CLS=8

Configure the modem for Voice Mode.

OK

The modem agrees, and is now set up for voice, but the DTE can change

things during the discrimination phase.

#BDR=16

The DTE knows that the compression requires a 38,400 bps speed, so it

forces this as the new speed.

OK

This OK message is sent at 9600 bps, but the modem then switches to

38,400 bps.  The S30 timer is now active.

#VSS=2

This enables default silence deletion.

OK

The modem confirms command reception.

A

The DTE answers.  (Note that this is where the DTE can let another

RING go by to process Caller ID.)

VCON

The modem is now in Online Voice Command Mode.  DTMF and calling tone

detection is enabled.  If fax or data calling tones are detected, the

DTE can change to Fax or Data Mode.

#VTX

The DTE immediately switches to Voice Transmit Mode to send out a

short greeting.

CONNECT

The modem says, "Send me the ADPCM data."

<Data>

The DTE sends a previously recorded message from a very fast talking

person: "Hello, this is Dave.  Press * to send a fax, # for help, or

leave your message at the beep." Note that the caller can enter DTMF

tones at any time, which the DTE can take into account, or calling

tone can still be detected.

<DLE><ETX>

This ends the greeting message.

VCON

The modem switches back to Online Voice Command Mode.

#VTS+#

The DTE sends # as the beep.

VCON

The modem is still able to receive DTMF or Calling tone here.

#VRX

Enter Receive Mode.  Now the modem is clocking silence, as well as

DTMF, and recording whatever the caller says.

<DLE>5

#VSP must have been set to 35, since 3.5 seconds later the modem

sends this indication of silence period expiration.  No ADPCM data was

sent to the DTE, because the data pump deleted all 3.5 seconds of

silence.  This tells the DTE that nobody is talking.

!

The DTE issues a key abort to switch to Online Voice Command Mode.

<DLE><ETX>

VCON

The modem exits Voice Receive Mode and switches to Online Voice

Command Mode.

#CLS=0

The DTE received no voice data, just the silence indicator after a

3.5 second wait.  Since there was no calling tone and the user never

entered any DTMF tones, the DTE assumes it must be a calling data

modem.  If the data handshake fails, the modem does not hang up the

line thus giving the DTE another chance to try the fax again (in this

case).

OK

The modem is now a data modem.

A

The modem pretends that it just got RING, try V.22 bis/V.42, etc.

Table 8-13.  Originate a Call, Send Answerer a Message

DTE to Modem

Modem to DTE

Description and Interpretation

#CLS=8

Configure the modem for Voice Mode.

OK

The modem agrees, and is now set up for voice.

#BDR=16

The DTE knows that compression requires a 38,400 bps speed, so it

forces this as the new speed.

OK

This OK message is sent at 2400 bps, but the modem then switches to

38,400 bps.

#VLS=0         (this ensures that the telephone line is selected)

OK

The modem agrees.

#VRA=45

The DTE allows 4.5 seconds for ringback to stop before assuming that

the modem thinks that the remote has answered.

OK

The modem agrees with the DTE's assumption.

DT6807

The DTE makes a telephone call, and the modem dials and goes through

call progress.  BUSY or NO DIALTONE can be detected, but in this

example, the modem gets ringback.

VCON

This message is sent 4.5 seconds after the third ringback cadence is

detected by the modem.  The modem is in effect assuming that the remote

telephone was answered on the third ring because no fourth ringback was

detected before the #VRA time-out.  The modem enters Online Voice

Command Mode, and DTMF and answer tone detection is enabled.  (Note

that during this period, the modem might have detected answer tone from

a data modem.  In this case, the appropriate <DLE> shielded code has to

be sent prompting the DTE to pursue a data handshake.)

#VTX

The DTE has a message to send.

CONNECT

The modem says: "send it."

<Data>

The DTE sends the message: "Hi Dave.  Your mother called."

<DLE><ETX>

Procedure complete.

VCON

DTMF still active.

H

The DTE has completed its forwarding task.